SPI and I2C Interview Preparation Handbook

Theory, waveforms, Linux APIs, driver concepts, debugging, and interview Q&A

How to use this document: Read the comparison and theory first, then practice explaining timing diagrams aloud. For coding interviews, focus on the user-space examples, kernel driver flows, error handling, and debugging sections.

# Table of Contents

High-level comparison

SPI theory and signaling

I2C theory and signaling

Linux user-space code examples

Linux kernel driver perspective

Device tree examples

Debugging and bring-up checklist

Common bugs and interview questions

# 1. High-Level Comparison

SPI and I2C are short-distance synchronous serial buses used to connect processors, microcontrollers, sensors, memories, PMICs, ADCs, DACs, displays, and board-management devices. Both use a clock line, but their topology, addressing model, electrical behavior, software model, and debugging failure modes differ significantly.

## Interview framing

SPI is simpler electrically and faster, but consumes more pins and lacks standard addressing.

I2C saves pins and supports many devices on two wires, but requires pull-ups, ACK handling, arbitration, and bus recovery knowledge.

When asked to choose one, discuss throughput, pin budget, device count, board length, power, latency, and driver support.

# 2. SPI Theory

SPI, Serial Peripheral Interface, is a synchronous serial bus. A controller drives the serial clock and selects one target using chip-select. Data shifts simultaneously from controller to target on MOSI and from target to controller on MISO.

SPI point-to-point bus

SPI bus with multiple chip-selects

## SPI terminology

## Clock polarity and phase

SPI mode is defined by CPOL and CPHA. CPOL controls idle clock level. CPHA controls which clock edge samples data.

SPI mode 0 timing sketch

## SPI transaction model

The controller asserts chip-select.

The controller clocks out command/address/dummy/data bytes.

Transmit and receive happen at the same time. Even a read operation transmits dummy bytes to generate clocks.

The controller deasserts chip-select to end the frame unless the protocol requires CS to stay active across multiple transfers.

SPI has no built-in ACK, address, retry, or error correction. Higher-level device protocol defines meaning.

## SPI advantages and limitations

# 3. I2C Theory

I2C, Inter-Integrated Circuit, is a two-wire synchronous serial bus using SCL for clock and SDA for bidirectional data. Devices share the same bus and are selected by an address transmitted in-band.

I2C shared bus

## Open-drain behavior

I2C devices do not actively drive a high level on SDA/SCL; they release the line and pull-up resistors bring it high.

Any device can pull the line low, which enables ACK, clock stretching, and arbitration.

Pull-up value affects rise time, power, and maximum speed. Too weak causes slow rising edges; too strong increases current and may exceed device drive capability.

## Start, stop, address, ACK

I2C basic write frame

I2C common register read using repeated start

## I2C addressing

7-bit addressing is most common. The byte on the wire is address shifted left by one plus the R/W bit.

Example: 7-bit address 0x48 appears as 0x90 for write and 0x91 for read on the bus.

10-bit addressing exists but is less common in board-level peripherals.

Address conflicts require alternate strap pins, multiplexers, or separate buses.

## Clock stretching and arbitration

Clock stretching lets a target hold SCL low to slow the controller while it prepares data.

Multi-master arbitration relies on open-drain signaling: a controller that releases SDA high but observes low loses arbitration.

Many embedded systems use single-master I2C, but interviewers may test whether you know arbitration exists.

## I2C advantages and limitations

# 4. Linux User-Space Code

## SPI using spidev

Linux exposes some SPI devices through /dev/spidevB.C when enabled by board configuration. The application configures mode, bits per word, and speed using ioctl(), then performs full-duplex transfers.

### SPI code talking points

SPI reads still transmit bytes because the controller must generate clocks.

rx[0] may be dummy or undefined depending on the device protocol.

Use SPI_IOC_MESSAGE(N) for one or more transfers under one chip-select assertion if cs_change is not set.

Mode mismatch is one of the most common causes of shifted or corrupted data.

## I2C using i2c-dev

Linux exposes I2C adapters through /dev/i2c-N when i2c-dev is enabled. Simple SMBus helpers work for many register devices; I2C_RDWR is needed for combined transactions with repeated start.

### I2C code talking points

Pass the 7-bit address to Linux APIs, not the left-shifted 8-bit address byte.

I2C_RDWR with two messages creates a write-register then repeated-start read transaction.

A NACK usually appears as an ioctl failure; confirm address, power, pull-ups, and bus number.

Some devices require delays after reset, conversion start, EEPROM write, or power mode changes.

# 5. Linux Kernel Driver Perspective

## SPI kernel driver skeleton

## I2C kernel driver skeleton

Kernel interview note: For real register-map devices, Linux drivers often use regmap. regmap abstracts register reads/writes over I2C, SPI, MMIO, and adds caching, locking, endianness handling, and debugfs visibility.

## regmap examples

# 6. Device Tree Examples

## SPI device tree

## I2C device tree

## Device tree interview checklist

For SPI, reg usually means chip-select number under the SPI controller node.

For I2C, reg is the 7-bit device address.

compatible controls driver matching.

spi-max-frequency protects the target from being clocked too fast.

clock-frequency on an I2C controller configures bus speed, not target address.

Pinmux, regulators, reset GPIOs, interrupts, and clocks often matter as much as the bus node.

# 7. Debugging and Bring-Up

## General workflow

Confirm power rails, reset pins, enable GPIOs, and reference clocks.

Confirm pinmux and board schematic match the software bus instance.

Check device tree compatible, address/chip-select, speed, and mode.

Use logic analyzer traces to verify wire-level behavior.

Lower bus speed during bring-up.

Check kernel logs with dmesg and driver probe messages.

Check /sys/bus/spi/devices, /sys/bus/i2c/devices, and relevant debugfs entries.

## SPI debugging checklist

No clock: controller not enabled, pinmux wrong, transfer not submitted, runtime PM issue.

No MISO: target not selected, wrong CS polarity, target in reset, MISO pin conflict.

Data shifted by one bit: wrong CPHA or sampling edge.

All 0xff: MISO pulled high or target not driving.

All 0x00: MISO pulled low, target held reset, or bus contention.

Works slow but fails fast: signal integrity, setup/hold timing, drive strength, trace length.

## I2C debugging checklist

No ACK: wrong 7-bit address, device unpowered, missing pull-ups, wrong bus, reset asserted.

SDA stuck low: target wedged mid-transaction, bad device, short, or missing recovery.

SCL stuck low: target clock stretching forever, short, controller issue.

Slow rise time: pull-ups too weak or bus capacitance too high.

Address confusion: datasheets sometimes show 8-bit write/read values instead of 7-bit address.

Intermittent errors: marginal pull-ups, long wires, noise, power sequencing, level shifting.

# 8. Common Interview Questions

## Conceptual questions

## Linux/driver questions

## Scenario questions

A sensor does not respond on I2C. Explain your debug flow from power to logic analyzer.

SPI flash returns 0xff for every byte. List possible root causes.

An I2C device works at 100 kHz but not 400 kHz. What do you check?

Two I2C devices have the same fixed address. What board/software options exist?

A SPI ADC returns data shifted by one bit. What configuration is suspicious?

A target holds SDA low after a reset. How can the controller recover the bus?

## Short answers to memorize

SPI selects by chip-select; I2C selects by address.

SPI is usually full-duplex; I2C is half-duplex.

I2C lines idle high because of pull-ups.

I2C ACK is the ninth clock bit.

Repeated start is essential for many register reads.

Wrong SPI CPOL/CPHA often looks like bit-shifted data.

Linux I2C addresses are normally written as 7-bit values.

regmap is preferred for many Linux register-based SPI/I2C drivers.

# 9. Quick Reference

# 10. Practice Prompts

Draw an I2C register read with repeated start and explain every ACK/NACK.

Draw SPI mode 0 timing and explain CPOL/CPHA.

Write pseudocode to read a two-byte register over I2C.

Explain why an SPI RX buffer changes even when you think you are only writing.

Explain how Linux binds a device tree node to an I2C or SPI driver.

Explain how you would recover an I2C bus with SDA stuck low.